MINISTRY OF PORTS SHIPPING AND WATERWAYS

NOTIFICATION

New Delhi, the xx March, 2026

Whereas, draft of the Merchant Shipping (Dumping of Waste and Other Matter at Sea) Rules, 2025 were published, as required by section xx of the Merchant Shipping Act, 2025 (No. 24 of 2025), vide G.S.R.__________, dated the ________[Month], 2025, inviting objections and suggestions from all persons likely to be affected thereby till the expiry of thirty days from the date of publication of the said notification in the Official Gazette;

And, whereas the copies of the said Gazette were made available to the public on the dd [Month], 2025; And whereas ‘n’ objection(s) and suggestion(s) have been received and considered on the said draft;

Now, therefore In exercise of the powers conferred by section 143, read with sections 131, 134 and 147 of the Merchant Shipping Act, 2025 ( of 2025) (hereinafter referred to as “the Act”), and in order to give effect to the International Convention for the Prevention of Pollution from Ships, 1973, as modified by the Protocol of 1978 relating thereto, Annex V, as amended from time to time (hereinafter “MARPOL Annex V”), the Central Government hereby makes the following rules, namely:—

[CHAPTER I — PRELIMINARY]

- Short title and commencement. (1) These rules may be called the Merchant Shipping (Prevention of Pollution by Garbage from Ships) Rules, 2025.
(2) They shall come into force on the date of their publication in the Official Gazette.

- Application. Unless expressly provided otherwise in these rules—

  - these rules apply to—

    - every Indian ship wherever it may be; and

    - every other ship while in any port, terminal, offshore facility, or maritime zones of India as defined in section 2 of the Territorial Waters, Continental Shelf, Exclusive Economic Zone and Other Maritime Zones Act, 1976; and

  - offshore platforms, fixed or floating, engaged in exploration, exploitation or associated processing of sea-bed mineral resources within areas under Indian jurisdiction.

  - These rules are in addition to, and not in derogation of, the Act and any other law for the time being in force.

- Definitions. (1) In these rules, unless the context otherwise requires—

    - “Act” means the Merchant Shipping Act, 2025;

    - “Administration” means the Government of India in the Ministry of Ports, Shipping and Waterways (Directorate General of Maritime Administration);

    - “Annex” means MARPOL Annex V, as amended;

    - “authorized officer” means any officer empowered under rule 19;

    - “cargo residues” means the remnants of any cargo which remain on deck or in holds following loading or unloading, including spillages and dust, not otherwise cleaned for reuse;

    - “e-waste” means electrical and electronic equipment and components from shipboard use that are disposed of;

    - “fishing gear” includes any physical device or part thereof used to capture marine life;

    - “garbage” has the meaning assigned in the Annex and includes all kinds of food, domestic and operational waste, plastics, dunnage, lining and packing materials, cargo residues, incinerator ashes, cooking oil, e-waste, operational wastes, animal carcasses and fishing gear;

    - “gross tonnage” means gross tonnage as determined under the International Convention on Tonnage Measurement of Ships, 1969;

    - “HME” (harmful to the marine environment) in relation to solid bulk cargoes has the meaning and criteria set out in Schedule III;

    - “MMD” means the Mercantile Marine Department of the Directorate General of Maritime Administration;

    - “place of reception” or “reception facility” means any facility adequate to receive ship-generated garbage without undue delay;

    - “special area” means any sea area designated as such under the Annex and listed in Schedule VI;

    - “Sensitive Area for Garbage Control (SAGC)” means an area declared under Rule 6(4);

    - Nearest land’ has the meaning assigned in the Annex, including the special baseline off the north-eastern coast of Australia.

  - Words and expressions used but not defined in these rules shall have the meanings respectively assigned to them in the Act or in the Annex.

- General obligation and prohibition

  - No owner, master or person in charge of a ship shall discharge or cause to be discharged any garbage into the sea except in accordance with these rules and the Annex.

  - Disposal of plastics, including synthetic ropes, nets and plastic garbage bags, and garbage mixed with plastics is prohibited in all areas.

  - Where garbage is mixed with other matter having different discharge requirements, the more stringent requirements shall apply.

  - Compliance with these rules is a condition for proceeding to sea and for entry into, or departure from, any Indian port or offshore terminal.

  - Except as expressly permitted by Rules 5 to 8, the discharge of any garbage into the sea is prohibited.

  - Irrespective of sea area, the discharge into the sea of cooking oil, domestic wastes, operational wastes, incinerator ashes, e-waste and fishing gear is prohibited.

- Discharge outside special areas (baseline regime). (1) Subject to rules 7 and 8, discharge into the sea outside special areas shall comply with the Annex and the following principles—

    - Food wastes: permitted only—
 (i) >12 nautical miles from the nearest land; or
 (ii) >3 nautical miles if comminuted or ground to pass a 25 mm screen, and in all cases the ship shall be en route and the discharge made as far as practicable from the nearest land.

    - Dunnage, lining and packing materials that float: permitted only >25 nautical miles from the nearest land and when en route.

    - Cargo residues (non-HME): discharge of residues and wash-water may be permitted outside special areas when the cargo is not HME, the residues do not contain plastics, appropriate reception facilities are not available or practicable, the ship is en route and the discharge is made as far as practicable from the nearest land, in accordance with the Annex and IMO guidelines.

    - Cleaning agents and additives: residues in wash-water may be discharged outside special areas only if such substances are not harmful to the marine environment as per Annex V guidelines.

    - Animal carcasses: only in accordance with IMO guidelines and at a maximum feasible distance from the nearest land.

- Discharge within special areas. (1) Within a special area listed in Schedule VI, the discharge of all garbage into the sea is prohibited except as permitted for food wastes that are comminuted or ground, and then only >12 nautical miles from the nearest land and when the ship is en route.

  - In the Antarctic area, the discharge of any garbage, including food wastes, is prohibited.

  - Nothing in this rule permits discharge of plastics or cargo residues that are HME.

  - Sensitive Areas for Garbage Control (SAGC)

    - The Directorate General of Maritime Administration (DGMA) may, on the recommendation of the Ministry of Environment, Forest and Climate Change (MoEFCC) or any of its designated statutory agencies, by notification through their Executive Order/Official Gazette, declare any area within India’s maritime zones to be a “Sensitive Area for Garbage Control (SAGC)” for the purposes of these rules.

    - A notification under clause (a) shall specify—

      - the geographical limits of the SAGC (by coordinates or chart references), including any seasonal or time-bound applicability;

      - the prescription for discharge applicable in the SAGC, which may include a zero-discharge standard (including food wastes) or any stricter restriction than those in this Rule;

      - any additional operational requirements, including mandatory retention onboard and delivery to reception facilities, record-keeping, and reporting obligations.

    - The DGMA may, from time to time, amend, suspend or withdraw a SAGC notification in consultation with MoEFCC, and shall ensure prompt promulgation through Notices to Mariners, NAVTEX/SafetyNET, and publication on official portals.

    - In cases of urgent environmental risk, the DGMA may issue an interim order declaring a temporary SAGC for a period not exceeding 90 days, pending formal notification under clause (a).

    - The discharge prescriptions notified for a SAGC shall not be less stringent than those applicable under this Rule for MARPOL special areas, and nothing in this sub-clause shall be construed to derogate from obligations under the Annex.

- Fixed or floating platforms and ships within 500 metres. (1) From fixed or floating platforms engaged in exploration, exploitation or associated offshore processing of sea-bed mineral resources, and from any other ship alongside or within 500 metres of such platforms, discharge of any garbage is prohibited.

  - Notwithstanding sub-rule (1), food wastes that are comminuted or ground may be discharged if the platform is >12 nautical miles from the nearest land.

- Exceptions. (1) Rules 5 to 7 shall not apply to:

    - disposal necessary for the safety of the ship or life at sea;

    - escape resulting from damage to a ship or its equipment provided all reasonable precautions were taken before and after the occurrence;

    - accidental loss of fishing gear, provided all reasonable precautions were taken to prevent such loss; and

    - fishing gear that poses a hazard to navigation or the marine environment, where prompt removal is not feasible; in such case, the master shall report the loss in accordance with rule 16(2).

- Placards. (1) Every ship of 12 metres or more in length overall shall display placards, in the working language of the ship’s personnel and also in English for international voyages, notifying the crew and passengers of the garbage discharge requirements applicable to the ship and its area of operation, in the form set out in Schedule II.

- Garbage Management Plan (GMP). (1) Every ship of 100 gross tonnage and above, and every ship certified to carry 15 or more persons, and every fixed or floating platform shall carry a Garbage Management Plan complying with the Annex and the guidelines referred to in Schedule II, Part B.

  - The Plan shall provide written procedures for collection, segregation, storage, processing and disposal of garbage, the use of equipment, and the designation of the person in charge.

  - The Plan shall be ship-specific, easy to understand, and kept in a language understood by the crew and, for international voyages, additionally in English.

  - The Administration may issue guidance for model plans.

- Garbage Record Book (GRB) and record-keeping (latest amendments applied). (1) Every ship of 100 gross tonnage and above, and every ship certified to carry 15 or more persons, and every fixed or floating platform, shall be provided with a Garbage Record Book in the format in Schedule I.

  - Ships <100 gross tonnage shall make entries relating to garbage discharges, deliveries to reception facilities, incineration, or accidental losses in the official log-book.

  - The GRB shall have two parts-

Part I (Categories A–I):

A- Plastics; B- Food wastes; C- Domestic wastes; D- Cooking oil; E- Incinerator ashes; F- Operational wastes; G- Animal carcasses; H- Fishing gear; I- E-waste; and;
Part II (Categories J–K):

J Cargo residues (non-HME); K Cargo residues (HME).

  - Every discharge, delivery to reception facility, or incineration shall be recorded and signed by the officer in charge, and each completed page shall be signed by the master; GRB pages shall be retained for two years after the last entry.

  - Electronic Record Books: The Administration may approve use of an electronic GRB meeting IMO performance, data integrity and auditability standards; where approved, the electronic GRB shall be treated as the GRB under these rules.

  - The competent authority at any Indian port may inspect and copy the GRB or log-book entries and require the master to certify the copies as true copies.

- Solid bulk cargoes: HME classification and shipper’s declaration (2018 amendment). (1) The criteria for determining whether a solid bulk cargo is HME are set out in Schedule III (adopted mutatis mutandis from the Annex).

  - The shipper shall, as part of the cargo information required under the Act and the IMSBC Code, declare whether the cargo is HME and provide supporting data.

  - Discharge at sea of cargo residues or wash-water from HME cargoes is prohibited in all areas.

  - When cargo residues (non-HME) or wash-water are to be discharged outside special areas, the conditions in rule 5(c) and applicable IMO guidelines shall be met.

- Disposal to and adequacy of reception facilities (1) Ports and terminals in India shall provide adequate reception facilities for ship-generated garbage without undue delay, commensurate with the types of ships normally using them.

  - Charges, if any, shall not be structured so as to discourage the use of reception facilities.

  - Allegations of inadequate facilities shall be reported by the master to the port and to the Administration in the format in Schedule V, for onward transmission to the IMO as appropriate.

  - Indian ships operating in Arctic waters may comply with regional reception arrangements recognized by the coastal State concerned, consistent with the Annex.

- Surveys, audits and verification (implementation of section 147). (1) Compliance with these rules shall be verified during statutory surveys, safety management audits and port State control inspections, as applicable under the Act.

  - No certificate is prescribed by the Annex; however, the Administration may issue a Statement of Compliance evidencing conformity with these rules upon satisfactory verification.

  - A ship shall not proceed to sea if it does not carry the placards, the Garbage Management Plan or the GRB/log-book entries as required by these rules.

- Port clearance and detention. (1) An authorized officer may withhold port clearance, or detain a ship, where there are clear grounds for believing the ship or its master/crew are not familiar with essential procedures relating to prevention of pollution by garbage, or where there is evidence of violation of these rules.

  - Directions for rectification, delivery of garbage to reception facilities, or off-loading prohibited wastes may be issued as conditions for release.

- Notifications and reports. (1) Discharges and incidents: The master shall record all discharges, deliveries and incinerations required by rule 11.

  - Lost fishing gear: Any loss of fishing gear that poses a hazard to navigation or the marine environment shall be reported without delay to the nearest coastal State authority and, when in Indian waters or by an Indian ship, to the Vessel Traffic Service/Indian Coast Guard and the Administration in the format in Schedule V.

  - Inadequate reception facilities: Reports shall be made per rule 13(3).

  - The Administration may prescribe digital portals for the submission of reports under this rule.

- Education, training and record retention. (1) The owner and master shall ensure crew training and familiarization on garbage management and recording procedures.

  - Records kept under these rules shall be readily available for inspection and retained for the period specified in rule 11(4).

- Incineration and on-board processing. (1) Incineration of garbage shall conform to applicable IMO requirements and must not include plastics.

  - Processing equipment (grinders, compactors, incinerators) shall be operated and maintained in accordance with manufacturer instructions and the GMP.

- Offences and consequences of contravention. (1) Any contravention of these rules constitutes an offence under the Act and is punishable in accordance with the provisions thereof[May specify the exact provision of the Act].

  - Without prejudice to any penalty under the Act, an authorized officer may direct operational restrictions, denial of entry or sailing, off-loading of garbage to reception facilities at the owner’s cost, and cost recovery for response actions under the Act.

- Enforcement agencies and powers. (1) For the purposes of these rules, the following are authorized officers when acting within their respective jurisdictional waters and ports:

    - Officers of the Directorate General of Maritime Administration and Mercantile Marine Department not below the rank notified by the Administration;

    - Officers of the Indian Coast Guard not below the rank of Assistant Commandant (or equivalent);

    - Officers of the Indian Navy not below the rank of Sub-Lieutenant (or equivalent) when duly assigned to maritime law-enforcement tasks by the Central Government;

    - Officers of the Border Security Force (Water Wing) not below the rank of Inspector in inland/coastal border waters;

    - Port officers and conservators of major and non-major ports to the extent empowered under port laws and these rules.

  - Authorized officers may board, inspect, search and examine any ship, require production of documents and records including the GRB and GMP, sample residues, seize evidence, record statements, and detain the ship for suspected violations.

  - Prosecutions may be instituted by or with the previous sanction of the Administration or by an authorized officer of the Indian Coast Guard, Indian Navy (when so assigned), BSF (Water Wing) or port authority, as the case may be.

  - The powers under this rule are in addition to the powers available under any other law for the time being in force.

- Power to issue directions and guidelines. (1) The Administration may issue directions, circulars or guidelines for uniform implementation of these rules, including approved electronic record books, model placards, GMP templates, criteria and testing methods for HME assessment, and reporting formats.

- Transitional provisions. (1) Ships shall update placards, Garbage Management Plans and GRBs to the formats required by these rules within six months from commencement.

  - Existing electronic record book approvals shall be revalidated to the extent necessary.

- Repeal and savings

(1) The Merchant Shipping (Prevention of Pollution by Garbage from Ships) Rules, 2009 are hereby repealed: Provided that anything done or any action taken thereunder, including any approval, direction, permission or order, shall be deemed to have been done or taken under the corresponding provisions of these rules, to the extent not inconsistent.

  - These rules shall be read harmoniously with Part VII of the Act, including sections 131, 134 and 147.

SCHEDULE I

(See rule 11)

Garbage Record Book — Format and Categories

The Annex-I to this rule provides the format of the Garbage Record Book containing the following;

Part I — Shipboard Garbage (Categories A–I)
A Plastics; B Food wastes; C Domestic wastes; D Cooking oil; E Incinerator ashes; F Operational wastes; G Animal carcasses; H Fishing gear; I E-waste.

For each discharge, delivery to reception facility, or incineration record: date/time; position/port; category; estimated amount (m³); method discharge/delivery/incineration); remarks; officer’s signature; master’s signature per page.

Part II — Cargo Residues (Categories J–K)
J Cargo residues (non-HME); K Cargo residues (HME).
Record the same particulars; for wash-water discharges, record start/stop positions, confirmation that cleaning agents/additives are not harmful, and unavailability/impracticability of reception facilities (if applicable).

Notes:

- Entries shall be in English for foreign-going ships (in addition to any other language).

- Retain GRB for two years after last entry.

- Electronic GRB permitted where approved by the Administration.

SCHEDULE II

(See rules 9 and 10)

Model Placard Text & Garbage Management Plan (Content)

A. Model Placard (indicative text)

- Disposal of plastics is prohibited everywhere.

- Dunnage/lining/packing that floats: only >25 nm from nearest land and when en route (not in special areas).

- Food waste: only >12 nm; or >3 nm if ground/comminuted (not in special areas); in special areas: only if ground/comminuted and >12 nm; in Antarctic area: prohibited.

- Cargo residues: discharge at sea prohibited if HME or containing plastics; non-HME residues/wash-water may be discharged outside special areas only when conditions are met.

- Keep Garbage Record Book; segregate garbage; deliver to reception facilities where required.

- Report lost fishing gear and any inadequate reception facilities.

B. Minimum Content of Garbage Management Plan

- Ship particulars; organization and responsible person.

- Garbage segregation plan and designated collection points.

- Processing & storage: compaction, comminution, grinding, incineration (no plastics), containers and labeling.

- Discharge/disposal procedures by category and sea area.

- Delivery to reception facilities and record-keeping.

- Training and familiarization; drills, internal checks and audits.

- Contingencies for accidental loss, damage, and reporting.

- Use of electronic GRB where approved.

SCHEDULE III

(See rule 12)

Criteria for “Harmful to the Marine Environment (HME)” for Solid Bulk Cargoes (extract)

A solid bulk cargo is HME if it meets any of the following (as per Annex V Appendix guidance):

- Acute aquatic toxicity: LC50 or EC50 ≤ 1 mg/L to fish/crustacean/algae;

- Chronic aquatic toxicity: NOEC/EC10 ≤ relevant thresholds or classification as Aquatic Chronic 1 or 2;

- Bioaccumulation: BCF ≥ 500 or log Kow ≥ 4;

- Persistence: Not rapidly degradable and meets specified half-life criteria;

- Other properties as may be specified by the Administration consistent with IMO guidance.
Note: Shipper’s declaration of HME status shall accompany cargo information.

SCHEDULE IV

(See rule 5(c))

Conditions for Discharge of Non-HME Cargo Residues and Hold-Wash Water (outside special areas)

- Ship is en route and discharge is made as far as practicable from nearest land.

- No plastics or other prohibited substances present.

- Hold-wash water contains no cleaning agents/additives that are harmful to the marine environment.

- Discharge only when reception facilities are not available or not practicable to use.

- Entries made in GRB Part II, including start/stop positions for wash-water.

SCHEDULE V

(See rules 13 and 16)

Reporting Formats

A. Inadequate Reception Facility Report (ship/port particulars; nature of inadequacy; date/time; impact on operations; copies to port and Administration).
B. Lost Fishing Gear Report (ship/fishing vessel particulars; position; description of gear; circumstances; hazard assessment; actions taken; contact details).

SCHEDULE VI

(See rule 3(1)(m) and rule 6)

Special Areas under Annex V (for ready reference)

- Mediterranean Sea area

- Baltic Sea area

- Black Sea area

- Red Sea area

- Gulfs area

- North Sea area

- Antarctic area (south of latitude 60° S)

- Wider Caribbean Region (including the Gulf of Mexico and the Caribbean Sea)

Explanatory Note (non-statutory): This draft transposes Annex V as revised, including (i) 2018 introduction of HME criteria and two-part GRB with E-waste and fishing gear categories, and (ii) 2024 extension of GRB to ≥100 GT and flexibility on Arctic reception arrangements for Indian ships operating there. It operationalizes sections 131, 134 and 147 of the Act and strengthens enforcement by expressly empowering Indian Coast Guard, Indian Navy (when assigned), BSF (Water Wing) and port authorities alongside DGMA/MMD.

Appendix I - Form of Garbage Record Book

Name of ship: _______________________ Distinctive number or letters: _______________________ IMO No.: _______________________ Period:_____________ From: _____________ To: _____________ 1. Introduction In accordance with regulation 9 of Annex V of the International Convention for the Prevention of Pollution from Ships, 1973, as modified by the Protocol of 1978 (MARPOL 73/78), a record is to be kept of each discharge operation or completed incineration. This includes discharges at sea, to reception facilities, or to other ships. 2. Garbage and garbage management Garbage includes all kinds of food, domestic and operational waste excluding fresh fish and parts thereof, generated during the normal operation of the vessel and liable to be disposed of continuously or periodically except those substances which are defined or listed in other annexes to MARPOL 73/78 (such as oil, sewage or noxious liquid substances). The Guidelines for the Implementation of Annex V of MARPOL 73/78* should also be referred to for relevant information. 3. Description of the garbage The garbage is to be grouped into categories for the purposes of this record book as follows: 1; Plastics. 2; Floating dunnage, lining, or packing material. 3; Ground-down paper products, rags, glass, metal, bottles, crockery, etc.. 4; Cargo residues, paper products, rags, glass, metal, bottles, crockery, etc.. 5; Food waste. 6; Incinerator ash.. 4 Entries in the Garbage Record Book 4.1 Entries in the Garbage Record Book shall be made on each of the following occasions: (a); When garbage is discharged into the sea:. (i); Date and time of discharge. (ii); Position of the ship (latitude and longitude). Note: for cargo residue discharges, include discharge start and stop positions.. (iii); Category of garbage discharged. (iv); Estimated amount discharged for each category in cubic metres. (v); Signature of the officer in charge of the operation.. (b); When garbage is discharged to reception facilities ashore or to other ships:. (i); Date and time of discharge. (ii); Port or facility, or name of ship. (iii); Category of garbage discharged. (iv); Estimated amount discharged for each category in cubic metres. (v); Signature of officer in charge of the operation.. (c); When garbage is incinerated:. (i); Date and time of start and stop of incineration. (ii); Position of the ship (latitude and longitude). (iii); Estimated amount incinerated in cubic metres. (iv); Signature of the officer in charge of the operation.. (d); Accidental or other exceptional discharges of garbage. (i); Time of occurrence. (ii); Port or position of the ship at time of occurrence. (iii); Estimated amount and category of garbage. (iv); Circumstances of disposal, escape or loss, the reason therefor and general remarks.. 4.2 Receipts The master should obtain from the operator of port reception facilities, or from the master of the ship receiving the garbage, a receipt or certificate specifying the estimated amount of garbage transferred. The receipts or certificates must be kept on board the ship with the Garbage Record Book for two years. 4.3 Amount of garbage The amount of garbage on board should be estimated in cubic metres, if possible separately according to category. The Garbage Record Book contains many references to estimated amount of garbage. It is recognized that the accuracy of estimating amounts of garbage is left to interpretation. Volume estimates will differ before and after processing. Some processing procedures may not allow for a usable estimate of volume, e.g. the continuous processing of food waste. Such factors should be taken into consideration when making and interpreting entries made in a record.

RECORD OF GARBAGE DISCHARGES Ship's name: _______________________ Distinctive No., or letters: _______________________ IMO No.: ____________ Garbage categories: .1; Plastic.. .2; Floating dunnage, lining, or packing materials.. .3; Ground paper products, rags, glass, metal, bottles, crockery, etc.. .4; Cargo residues, paper products, rags, glass, metal, bottles, crockery, etc.. .5; Food waste.. .6; Incinerator ash except from plastic products which may contain toxic or heavy metal residues.. NOTE: THE DISCHARGE OF ANY GARBAGE OTHER THAN FOOD WASTE IS PROHIBITED IN SPECIAL AREAS. ONLY GARBAGE DISCHARGED INTO THE SEA MUST BE CATEGORIZED. GARBAGE OTHER THAN CATEGORY 1 DISCHARGED TO RECEPTION FACILITIES NEED ONLY BE LISTED AS A TOTAL ESTIMATED AMOUNT. DISCHARGES OF CARGO RESIDUES REQUIRE START AND STOP POSITIONS TO BE RECORDED. Date/time; Position of the ship; Estimated amount discharged into sea (m3); Estimated amount discharged to reception facilities or to other ship (m3); Estimated amount incinerated (m3); Certification/ Signature. Cat. 2; Cat. 3; Cat. 4; Cat. 5; Cat. 6; Cat. 1; Other. * Refer to the Guidelines for the Implementation of Annex V of MARPOL 73/78, as amended by resolutions MEPC.59(33) and MEPC.92(45)..